# Wind Sensor Data vs Real-Time Comparison

Tirunelveli Region  |  2026-03-08 at 08:49 UTC  |  Open-Meteo API

sensors: 8.71°N/77.76°E · 8.95°N/77.49°E · 8.50°N/78.05°E

## 1. U/V Component Math Verification

All three sensors compute u and v wind components using the standard meteorological formula: u = −speed·sin(dir), v = −speed·cos(dir). km/h conversion uses ×3.6. All values below verified to 4 decimal places.

| Sensor | Speed (m/s) | Dir (°) | u calc | v calc | km/h | Gust (m/s) | Speed (m/s) | ✓ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sensor_1 | 4.69 | 79° | -4.69·sin79° = -4.6038 | -4.69·cos79° = -0.8949 | 16.884 | 8.8 | 4.69 | ✅ |
| sensor_2 | 4.40 | 69° | -4.40·sin69° = -4.1078 | -4.40·cos69° = -1.5768 | 15.84 | 8.2 | 4.40 | ✅ |
| sensor_3 | 4.71 | 86° | -4.71·sin86° = -4.6985 | -4.71·cos86° = -0.3286 | 16.956 | 9.4 | 4.71 | ✅ |

## 2. Real-Time Field Comparison

Sensor readings at 14:15 IST compared against live ground observations and cross-referenced sources (Windy.app, Weather.com, Open-Meteo current endpoint).

| Sensor | Field | Sensor Data | Real-Time | Delta | Status | Note |
| --- | --- | --- | --- | --- | --- | --- |
| sensor_1 | Wind Speed | 4.69 m/s | 3.7 m/s | +0.99 m/s | ⚠️ | Acceptable range |
| sensor_1 | Wind Dir | 79° (ENE) | ~180° (S) | ~101° off | ❌ | Model lag |
| sensor_1 | Temp | 35.2°C | 35°C (feels like) | ~0.2°C | ✅ | Matches feels-like |
| sensor_1 | Humidity | 35% | 61% | -26% | ❌ | Major underest. |
| sensor_2 | Wind Speed | 3.09 m/s | ~2–4 m/s | In range | ✅ | Plausible |
| sensor_2 | Wind Dir | 60° (NNE) | West (~270°) | ~210° off | ❌ | Model lag |
| sensor_2 | Temp | 36.5°C | 35.9°C | +0.6°C | ✅ | Good match |
| sensor_2 | Humidity | 32% | ~60% | -28% | ❌ | Major underest. |
| sensor_3 | Wind Speed | 4.71 m/s | N/A (coastal) | — | ⚠️ | No ground truth |
| sensor_3 | Wind Dir | 86° (E) | SE–S (coastal) | ~60° off | ⚠️ | Partly plausible |
| sensor_3 | Temp | 33.8°C | 31.6°C | +2.2°C | ⚠️ | Coastal variation |
| sensor_3 | Humidity | 50% | 65%+ (coast) | -15% | ⚠️ | Low for coastal |

## 3. Summary Findings & Root Causes

| Parameter | sensor_1 | sensor_2 | sensor_3 | Root Cause | Fix |
| --- | --- | --- | --- | --- | --- |
| U/V Math | ✅ | ✅ | ✅ | — | None needed |
| Wind Speed | ⚠️ | ✅ | ⚠️ | OWM/Open-Meteo model resolution ~1–2km; localised variation | Accept ±1 m/s tolerance |
| Wind Direction | ❌ | ❌ | ⚠️ | Open-Meteo NWP model has 15–30 min lag; diurnal direction shifts not captured | Use hourly forecast field + interpolate |
| Temperature | ✅ | ✅ | ⚠️ | Sensor_3 coastal location has sea-breeze cooling not in model | Add coastal offset correction |
| Humidity | ❌ | ❌ | ⚠️ | Open-Meteo 'current' humidity may be forecast-interpolated, not observed | Query hourly=relative_humidity_2m at current hour |
| Gust > Speed | ✅ | ✅ | ✅ | Validation fix applied in collector script | Already fixed |

## 4. Recommendations

Wind Direction: Switch from Open-Meteo 'current' endpoint to the 'hourly' endpoint querying wind_direction_10m at the current UTC hour. This avoids NWP lag and returns the most recently run forecast slice.

Humidity: Replace current.relative_humidity_2m with hourly=relative_humidity_2m&forecast_days=1 and extract the value at the current hour index. The current field appears to interpolate from a prior model run.

Temperature (sensor_3): Apply a coastal correction factor of −1.5°C to −2°C for the Tiruchendur sensor (8.5°N, 78.05°E) to account for sea-breeze influence not captured by the 1km grid.

Validation: Implement a post-collection sanity check that compares sampled direction against the previous reading. Flag any delta >90° within 15 minutes as a potential model cache hit rather than a true wind shift.

Report auto-generated | ECE Climate Forecasting Hackathon 2026